SDK Manual

目录

1. File Direcotory specification	2

2. Use guide	2

2.1. First use	2

2.2. OposCashDrawer SO has been installed before	2

3. Develpment process	3

4. Related SDK Specfication	5

- File Direcotory specification

| File Name | Description |
| --- | --- |
| cash_drsow_so.dll | OPOS Cash Drawoer SO file |
| GPIOLib.dll | GPIO Service Interface |
| GPIOLib.pdb | GPIO Service pdb file |
| Demo_src | OPos cash Drawer demo source code |

- Use guide

  - First use

According to《Installation instruction.docx》to install opos cash drawer driver，steup program will install and regisit opos automatically, then you can develop your opos program。Demo directory is the demo program.

  - OposCashDrawer SO has been installed before

- Copy to CashDrawer SO file directory.

- Add SO register iitem to windows register table, as below：

- Reb box content:

- MJ_CashDrawer: 				CashDrawer’s name

- OPOS.CashDrawer.MeKing.1：   CashDrawer’s regist information

- Blue Box content：

- Service item

- Value：C:\Program Files\OPOS\SOCommon\cash_draw_so.dll

- SO driver location(accord to you real location and modify this valu)

- （if you want modify this value to youself defined name, please contact SO vendor）

- Develpment process

- Develop process as below：

- According to figure：

- Invoke the function:  Open()->ClaimDevice()->DeviceEnable(). After that, the cash drawer service object will be opened, exclusive and enabled。

- Call OpenDrawer() to open cash drawer. If OpenDrawer() returned success, the drawer will be opened  and statusChangeEvent will be triggered to notify cash drawer is opened。

- Call waitForDrawerClose() to wait cash drawer is closed. If cash drawer is not closed in time, the system buzzer will be alarmed.

- Call waitForDrawerClose(). The function will not return until user close the cash drawer or the cash is not been opend yet.

- Related SDK Specfication

long Open(LPCTSTR DeviceName)

Desc：Open OPOS Control object

Params：deviceName, such as ＂MK_CashDrawer＂

Return：0=success, other value means failed

long get_OpenResult()

Desc：get open() call result

Params：None

Return：0=success, other value means failed

long ClaimDevice(long Timeout)

Desc：claim Opos device and make it exclusive

Params：timeOut value(ms)，

0：try to claimed device immediately, function will return immediately;

greater than 0 (unit ms)，exclusive device within the specified timeout range (ms);

-1: Waiting for exclusive device to release, if the device is monopolized by another 	device, wait for the device to be released continuously。

Return：0=success, other value means failed

BOOL get_Claimed()

Desc：get claimed status of the Opos Service Object

Params：None

Return：ture: device is exclusive；false: device is free

long ReleaseDevice()

Desc：release exclusive deivce

Params：None

Return：0=success, other value means failed

long Close()

Desc：Close Opos Device

Params：None

Return：0=success, other value means failed

void put_DeviceEnabled(BOOL newValue)

Desc：change to Opos device is Enable or Disable

Params：true= enable，false=disable

Return：None

BOOL get_DeviceEnabled()

Desc：Obtain device enable status

Params：None

Return：ture:device is enabled；false: device is disabled

BOOL get_FreezeEvents()

Desc：Obtain whether the device event notification status has been frozen (event notifications are not accepted in frozen state)

Params：None

Return：ture: Event notification has been frozen ；false: Event notification has not been frozen

void put_FreezeEvents(BOOL newValue)

Desc：Set device event notification freeze status

Params：true=freeze Event notification，false=trigger Event notification

Return：None

long get_ResultCode()

Desc：Get the function execution result of the function

Params：None

Return：0=success, other value means failed

long get_ResultCodeExtended()

Desc：Get the function execution extension result of the function

Params：None

Return：0=success, other value means failed

CString get_ControlObjectDescription()

Desc：get OPOS Control Object description information

Params：None

Return：Control Object description String.

long get_ControlObjectVersion()

Desc：get OPOS Control Object version information

Params：None

Return：version information

CString get_ServiceObjectDescription()

Desc：get OPOS Service Object description information

Params：None

Return：Service Object description String

long get_ServiceObjectVersion()

Desc：get OPOS Service Object version information

Params：None

Return：version information

CString get_DeviceDescription()

Desc：get OPOS device description

Params：None

Return：device description

CString get_DeviceName()

Desc：get device’s name

Params：None

Return： the name of device。

long CheckHealth(long Level)

Desc： check device’s health status

Params：	0=Internal Healch check

1==External Healch check

2==Interactive Healch check

Return：0=success, other value means failed

CString get_CheckHealthText()

Desc：get CheckHealth() result string

Params：None

Return：heath text string

BOOL get_DrawerOpened()

Desc：get cash drawer is opened or not

Params：None

Return：ture:Open；false: close

long OpenDrawer()

Desc：Open the cash drawer

Params：None

Return：0=success, other value means failed

long WaitForDrawerClose(long BeepTimeout, long BeepFrequency, long BeepDuration, long BeepDelay)

Desc：Waiting for the cash box to close; If the cash box has been closed, return immediately; otherwise, sound the buzzer alarm

Params：	BeepTimeOut， Buzzer alarm time out

BeepFrequency, Buzzer alarm frequency

BeepDruation,   Buzzer alarm duration

BeepDelay,  next Buzzer alarm dealy time

Return： if cash drawer is closed, function return immediately, otherwire will wait for cash drawer is closed, and then return.

0=success, other value means failed